New USB firmware Upgrade Operation

1, Connect the micro USB port to the PC, then reboot the unit, you will see a USB disk found in PC as below, that means boot loader work correctly. If not, pls check the connection/setting and upload the file again.

2. Open the disk, you will see a file named “Ready”, copy the firmware file to the folder and an auto update process will happen (the disk will refresh and the file name change to “Success”, this means the firmware is updated successfully). If not this name, pls check the file and connection then try this step again.

2. After the firmware update has finished, disconnect the USB cable, then reboot the unit again.